KMORFS

Kinetic Model of Residual Film Stress

User Manual — Python Edition

Produced by Chason Research Group
Brown University

Version 1.0  ·  2025

# Table of Contents

1.  Introduction  ..........................................  3

2.  System Requirements and Installation  ..................  3

3.  Opening the GUI  .......................................  4

4.  Overview of the Four Modes  ............................  5

5.  Mode 1 — General Stress-Thickness Fitting  .............  6

5.1  What Mode 1 Does  .....................................  6

5.2  Preparing Your Data  ..................................  6

5.3  Setting Up mainfile.xlsx  .............................  7

5.4  Running Mode 1 from the GUI  ..........................  8

5.5  Understanding the Output  .............................  8

6.  Mode 2 — Incremental (Steady-State) Stress Fitting  ....  9

6.1  What Mode 2 Does  .....................................  9

6.2  Preparing Your Data  ..................................  9

6.3  Setting Up mainfile.xlsx  .............................  10

6.4  Running Mode 2 from the GUI  ..........................  10

6.5  Understanding the Output  .............................  11

7.  Mode 3 — Alloy Extension Fitting  ......................  11

7.1  What Mode 3 Does  .....................................  11

7.2  Setting Up mainfile.xlsx  .............................  12

7.3  Running Mode 3 from the GUI  ..........................  12

8.  Mode 4 — Early-State Nucleation Fitting  ...............  13

8.1  What Mode 4 Does  .....................................  13

8.2  Setting Up mainfile.xlsx  .............................  13

8.3  Running Mode 4 from the GUI  ..........................  14

9.  Output Files and Plots  ................................  14

10.  Troubleshooting  ......................................  15

11.  Appendix — Physics Background  ........................  16

# 1.  Introduction

This document is the user manual for the Python edition of KMORFS (Kinetic Model of Residual Film Stress). KMORFS fits experimental thin-film stress data to a physics-based kinetic model using non-linear least-squares optimisation. It is produced by the Chason Research Group at Brown University.

This edition of KMORFS is written entirely in Python and replaces the original MATLAB version. It is launched through a graphical interface (GUI) with four buttons — one for each fitting mode. You do not need to write any code to use KMORFS.

The four modes are:

Mode 1 — General Stress-Thickness Fitting  (pure metals, multiple datasets)

Mode 2 — Incremental / Steady-State Stress Fitting  (rate-dependent SSSF data)

Mode 3 — Alloy Extension Fitting  (binary alloys via rule of mixtures)

Mode 4 — Early-State Nucleation Fitting  (nucleation / coalescence regime)

# 2.  System Requirements and Installation

Required software:

Windows 10 or later  (the GUI uses the Windows Tkinter display)

Anaconda (or Miniconda) with a Python 3.9+ environment

The following Python packages: torch, numpy, pandas, matplotlib, scikit-learn, scipy, openpyxl

Installation steps:

Step 1.  Install Anaconda from https://www.anaconda.com if you have not already done so.

Step 2.  Open the Anaconda Prompt (search for it in the Windows Start menu).

Step 3.  Create or activate the environment you want to use:

conda activate data2060new

Step 4.  Clone or download the KMORFS repository from GitHub:

git clone https://github.com/tongsu-brown/KMORFS.git

Step 5.  Navigate into the repository folder:

cd KMORFS

Step 6.  Install the required Python packages:

pip install -r requirements.txt

Note: If you see an error about OpenMP (OMP Error #15), this is normal on Windows with Anaconda. The GUI handles this automatically by setting KMP_DUPLICATE_LIB_OK=TRUE before launching each script.

# 3.  Opening the GUI

The GUI is a single Python file called kmorfs_gui.py located in the root of the KMORFS folder. You can launch it in two ways:

Option A — From Anaconda Prompt (recommended):

Step 1.  Open the Anaconda Prompt.

Step 2.  Activate your Python environment:

conda activate data2060new

Step 3.  Navigate to the KMORFS folder:

cd D:\KMORFS_local\KMORFS-db

Step 4.  Launch the GUI:

python kmorfs_gui.py

Step 5.  The KMORFS window will appear on your screen (see Figure 1 below).

Option B — Double-click (Windows):

Step 1.  Right-click on kmorfs_gui.py in Windows Explorer.

Step 2.  Select "Open with" → "Python" (make sure it points to your Anaconda environment).

What you will see:

The GUI window is divided into three sections:

Header — the KMORFS title and Brown University affiliation.

Four mode cards — each card shows the mode name, a short description, and three buttons:  Run,  Stop,  and  Folder.

Output Log — a dark terminal area at the bottom that streams live text output from any running script.

The status label on each card changes to reflect what is happening:  Idle  means the mode is not running,  Running...  means it is active,  Done  means it finished successfully,  and  Error  means something went wrong.

Note: You can run multiple modes at the same time. Each mode runs independently in its own background thread.

# 4.  Overview of the Four Modes

| Mode | Colour | Best for | Physics | Needs PyTorch? |
| --- | --- | --- | --- | --- |
| Mode 1 General Stress-Thickness | Blue | Pure metals, multi-dataset fitting | Growth kinetics + grain growth + energetic | Yes |
| Mode 2 Incremental Stress | Green | Rate-dependent steady-state stress (SSSF) | Growth kinetics + energetic | No |
| Mode 3 Alloy Extension | Amber | Binary alloys (e.g., Cr-W, V-Mo) | Rule-of-mixtures blending of pure elements | Yes |
| Mode 4 Early-State Nucleation | Violet | First ~20 nm of growth (Ag, Au, Ni) | Ellipsoidal grain-cap geometry + dA/dt | No |

Each mode reads its configuration from a file called mainfile.xlsx located inside the mode's own subfolder. You edit this file to point to your experimental data and to set initial parameter guesses before running.

5.  Mode 1 — General Stress-Thickness Fitting

## 5.1  What Mode 1 Does

Mode 1 fits experimental stress-thickness curves (stress × film thickness vs. film thickness) for one or more pure metals simultaneously. It uses a PyTorch-based optimiser to find the best material parameters (Sigma0, BetaD, Ea, Mfda, Di, A0, B0, l0) that are shared across all datasets of the same material, while process parameters (SigmaC, K0, alpha1, L0, GrainSize_200) can vary per dataset.

## 5.2  Preparing Your Data

Your experimental data must be in the KMORFS database before running Mode 1. The database consists of two CSV files inside the data/ folder:

data/source.csv  — one row per experiment, storing metadata.

data/all_experiments.csv  — the actual thickness / stress-thickness data points.

To add your own data:

Step 1.  Open data/source.csv in Excel.

Step 2.  Add one new row per experiment with the following columns:

| Column | Meaning | Example |
| --- | --- | --- |
| material | Material name | Cu |
| Tm_K | Melting temperature (K) | 1358 |
| data_source | Label for your data source | MyLab |
| R | Deposition rate (nm/s) | 0.1 |
| T | Temperature (K) | 295 |
| P | Pressure (Pa) — 0 for evaporation | 0 |
| Alloy_type | Single element or alloy name | Single element |

Step 3.  Open data/all_experiments.csv in Excel.

Step 4.  Add rows for each measured data point:

| Column | Meaning | Units |
| --- | --- | --- |
| material | Same material name as in source.csv | — |
| data_source | Same source label | — |
| R | Deposition rate | nm/s |
| T | Temperature | K |
| P | Pressure | Pa |
| thickness | Film thickness at this data point | nm |
| stressthickness | Measured stress-thickness | N/m |

Note: Each experiment should have at least 4 data points for a reliable fit.

## 5.3  Setting Up mainfile.xlsx

The file general_stress_thickness/mainfile.xlsx controls which materials to fit and provides the initial parameter guesses. Open it in Excel.

The spreadsheet has the following layout:

| Row | Column A | Columns 3–11 (parameters) |
| --- | --- | --- |
| Row 2 | Column header names | Parameter names: Sigma0, BetaD, Ea, Mfda, Di, A0, B0, l0, SigmaC |
| Row 3 | 0=shared across datasets, 1=per-dataset | 0 means the same value is used for all datasets of this material |
| Row 4 | Bound type (3=add, 4=multiply, 5=fix-at-zero) | Controls how the optimiser search range is computed |
| Row 5 | Bound magnitude | How far the parameter is allowed to vary from its initial guess |
| Row 6+ | Material name ¦ filename | Initial parameter values for each dataset |

Key things to change before your first run:

Step 1.  In rows 6 and below, replace the material name in column A with your material (e.g., "Cu").

Step 2.  Replace the filename in column B with the data file name as it appears in source.csv.

Step 3.  Enter your best initial guess for each parameter in the corresponding column.

Step 4.  Save and close mainfile.xlsx before clicking Run.

Note: You can also configure the materials list at the top of fit_general_stress.py by editing the MATERIALS and DATA_SOURCES variables. This lets you filter which entries from the database are used.

## 5.4  Running Mode 1 from the GUI

Step 1.  Launch the GUI (see Section 3).

Step 2.  Click the blue Run button on the Mode 1 card.

Step 3.  Watch the Output Log at the bottom of the GUI. You will see iteration numbers and cost values printing in real time.

Step 4.  When the fitting is complete, the card status changes to Done (tick mark in green).

Step 5.  Click the Folder button on the Mode 1 card to open the general_stress_thickness/ folder in Windows Explorer.

Note: To stop a fit early, click the red Stop button on the Mode 1 card.

## 5.5  Understanding the Output

Results are saved to the general_stress_thickness/output/ folder:

optimized_parameters.csv — the best-fit parameter values for each dataset and material.

fitting_result.jpg — a publication-quality plot (300 DPI) comparing the model (solid lines) to your data (symbols). Each dataset is shown in a different colour.

6.  Mode 2 — Incremental (Steady-State) Stress Fitting

## 6.1  What Mode 2 Does

Mode 2 fits steady-state stress-free (SSSF) data — measurements of the steady-state stress as a function of deposition rate for different material compositions. It does not require PyTorch and uses SciPy L-BFGS-B optimisation with a NumPy batch stress equation. It is well suited for multi-composition systems such as nitride alloys (e.g., Ti-Zr-N).

## 6.2  Preparing Your Data

Mode 2 reads from a CSV file (not the shared database). The CSV should have the following columns:

| Column | Meaning | Units |
| --- | --- | --- |
| R | Deposition rate | nm/s |
| T | Temperature | K |
| P | Pressure (0 for non-energetic deposition) | Pa |
| stress | Measured steady-state stress | GPa |
| composition | Alloy composition value (0=pure A, 1=pure B, 0.5=50/50) | — |

Step 1.  Save your data as a CSV file (e.g., MyMaterial.csv) and place it in the incremental_stress/ folder.

## 6.3  Setting Up mainfile.xlsx

Open incremental_stress/mainfile.xlsx in Excel. The layout is:

| Row | Content |
| --- | --- |
| Row 2 | Parameter names: composition, sigma0, betaD, diffusivity, p0, aprime, bprime, lprime, grainSize, data_file, material_map |
| Row 3 | 0=alloy-blended (interpolate from pure elements), 1=independent per composition |
| Row 4 | Lower bound for each parameter |
| Row 5 | Upper bound for each parameter |
| Row 6+ | One row per composition: initial guesses, data_file name, material_map string |

The material_map column tells the program which composition value corresponds to each label in the CSV. For example:

TiN=0;ZrN=1;TiZrN2=0.5

This means TiN is pure Ti side (0), ZrN is pure Zr side (1), and TiZrN2 is the 50/50 alloy (0.5).

Step 1.  Replace the data_file value with your CSV filename.

Step 2.  Update the material_map to match the column labels in your CSV.

Step 3.  Adjust initial parameter guesses and bounds.

Step 4.  Save mainfile.xlsx before clicking Run.

## 6.4  Running Mode 2 from the GUI

Step 1.  Click the green Run button on the Mode 2 card.

Step 2.  Watch the Output Log for iteration progress.

Step 3.  When done, click Folder to open the incremental_stress/ folder.

## 6.5  Understanding the Output

incremental_stress/output/optimized_parameters.csv — best-fit parameters per composition.

incremental_stress/output/fitting_result.jpg — plot of steady-state stress vs. deposition rate.

7.  Mode 3 — Alloy Extension Fitting

## 7.1  What Mode 3 Does

Mode 3 fits stress-thickness data for binary alloy systems. The energetic parameters (A0, B0, l0) for any alloy composition are automatically interpolated from the fitted pure-element values using a rule-of-mixtures approach. This means you must include the pure elements (e.g., Cr and W) before the alloys (e.g., Cr-25W, Cr-50W) in the materials list.

Note: IMPORTANT: Always list pure elements before alloys in the MATERIALS configuration. The program uses the pure-element fits to compute alloy parameters automatically.

## 7.2  Setting Up mainfile.xlsx

Open alloy_extension_stress_thickness/mainfile.xlsx. The layout is similar to Mode 1 with these differences:

| Parameter | Meaning in Mode 3 |
| --- | --- |
| alpha1, L0, GrainSize_200 | Grain size parameters — shared per material |
| Sigma0, BetaD, Mfda, Di | Kinetic parameters — shared per material |
| A0, B0, l0 | Energetic parameters — automatically blended for alloys |
| K0 (process) | Starting stress-thickness — one value per dataset |
| SigmaC and Ea | Fixed at zero in Mode 3 (not fitted) |

Step 1.  List pure elements first in the spreadsheet rows (e.g., Cr datasets in rows 6–10, W datasets in rows 11–15).

Step 2.  Then list alloy datasets below (e.g., Cr-25W in rows 16–18).

Step 3.  Enter initial guesses for pure-element parameters in the first row of each material.

Step 4.  For alloy rows, only the K0 value needs an initial guess; the energetic parameters will be computed automatically.

Step 5.  Save mainfile.xlsx before clicking Run.

## 7.3  Running Mode 3 from the GUI

Step 1.  Click the amber Run button on the Mode 3 card.

Step 2.  Watch the Output Log. You will see fitting progress for pure elements first, then alloys.

Step 3.  Click Folder to open alloy_extension_stress_thickness/output/.

optimized_parameters.csv — fitted parameters for all pure elements and alloys.

fitting_result.jpg — stress-thickness plot for all compositions.

8.  Mode 4 — Early-State Nucleation Fitting

## 8.1  What Mode 4 Does

Mode 4 fits stress-thickness data in the very early stage of thin-film growth — typically the first 5–20 nm — during which grains are still nucleating and coalescing. This is fundamentally different physics from Modes 1–3: grains are modelled as ellipsoidal caps, and stress arises from grain boundary area derivatives (dA/dt) rather than from surface energy or pressure effects.

Currently supported materials:

| Material | Datasets | Temperature range | Typical final cost |
| --- | --- | --- | --- |
| Ag (silver) | 9 curves | 193–373 K (−80°C to 100°C) | ~12 |
| Au (gold) | 4 curves | 303–523 K (30°C to 250°C) | ~2 |
| Ni (nickel) | 6 curves | 300–473 K (27°C to 200°C) | ~3 |

## 8.2  Setting Up mainfile.xlsx

Each material has its own mainfile.xlsx inside early_state_stress_thickness/data/{Ag,Au,Ni}/. Open the one for your chosen material.

| Row | Column A (data file) | Other columns (parameters) |
| --- | --- | --- |
| Row 2 | — | Parameter names: SigmaC, Sigma0, BetaD, Ea, Mfda, L_nul, K0, alpha2 |
| Row 3 | — | 0=shared across all datasets, 1=per-dataset, 3=special type |
| Row 4 | — | Variability (controls search range) |
| Row 5+ | Excel data filename | Initial parameter guesses for each dataset |

To run with the built-in Ag/Au/Ni data, no changes are needed — just click Run.

To add your own early-state data:

Step 1.  Create a new subfolder inside early_state_stress_thickness/data/ with your material name.

Step 2.  Copy a mainfile.xlsx from an existing material as a template.

Step 3.  Add your Excel data files (one per experimental curve) to the folder.

Step 4.  Edit mainfile.xlsx: list your data filenames in column A and set initial parameter guesses.

Step 5.  At the top of fit_early_state_stress.py, set the MATERIAL variable to your folder name:

MATERIAL = "MyMaterial"

## 8.3  Running Mode 4 from the GUI

Step 1.  Click the violet Run button on the Mode 4 card.

Step 2.  Mode 4 runs three rounds of optimisation (you will see Round 1/2/3 in the Output Log).

Step 3.  The fit typically takes 30–120 seconds depending on the material and number of datasets.

Step 4.  Click Folder to open early_state_stress_thickness/output/.

optimized_parameters.csv — fitted parameters for each dataset.

fitting_result.jpg — stress-thickness curves with model overlaid.

# 9.  Output Files and Plots

All modes save their results to an output/ subfolder inside the mode's working directory. For example, Mode 1 outputs go to general_stress_thickness/output/.

| File | Contents |
| --- | --- |
| optimized_parameters.csv | Best-fit values for all parameters, one row per dataset |
| fitting_result.jpg | 300 DPI publication-quality plot; symbols = data, lines = model fit |

About the plot:

X-axis: film thickness (nm) for Modes 1, 3, 4;  deposition rate (nm/s) for Mode 2.

Y-axis: stress-thickness (N/m) for Modes 1, 3, 4;  steady-state stress (GPa) for Mode 2.

Each colour corresponds to a different dataset or deposition condition.

Solid lines are the model fit; symbols (circles/squares) are the experimental data.

Note: You can click the Folder button on any mode card to open its working directory directly in Windows Explorer without leaving the GUI.

# 10.  Troubleshooting

| Problem | Likely cause | Solution |
| --- | --- | --- |
| GUI does not open / Python error on launch | Wrong Python interpreter | Edit the PYTHON variable at the top of kmorfs_gui.py to point to your Anaconda environment's python.exe |
| Output Log shows OMP Error #15 | OpenMP DLL conflict in Anaconda | The GUI already sets KMP_DUPLICATE_LIB_OK=TRUE automatically. If running scripts directly, add os.environ['KMP_DUPLICATE_LIB_OK']='TRUE' at the top. |
| Error (rc=1) on Mode 1 or 3 card | mainfile.xlsx still open in Excel | Close mainfile.xlsx in Excel before clicking Run |
| Script runs but the fit looks poor | Initial parameter guesses are far from the true values | Consult the README or the Appendix for typical parameter ranges per material. Try adjusting Sigma0 and BetaD first. |
| No data found for material X | Material name in mainfile.xlsx does not match source.csv | Check that the material name is spelled exactly the same in both files (case-sensitive) |
| Mode 4 output cost is very large (> 100) | Initial L_nul or Sigma0 far from optimal | Refer to the paper parameters in the README memory notes: Ag Sigma0~5, Ni Sigma0~1 (very low), Au L_nul~50 nm |
| Plots do not appear | Backend issue | The GUI does not suppress matplotlib windows. If plots still don't appear, restart the GUI. |

# 11.  Appendix — Physics Background

This appendix provides a brief description of the physics equations used in each mode. For full derivations, refer to the published papers listed in the References section.

## A.1  The General Stress Equation (Modes 1–3)

The model computes the instantaneous rate of change of stress-thickness (normalised curvature) with film thickness as the sum of three terms:

Term 1 — Growth kinetics: tensile stress from new grain boundary formation minus compressive stress from atomic diffusion into grain boundaries. Controlled by Sigma0, BetaD, Ea, SigmaC.

Term 2 — Grain growth stress: stress created as buried grains grow laterally. Controlled by Mfda, alpha1, L0, GrainSize_200.

Term 3 — Energetic (sputtering) effects: additional compressive stress from energetic ion bombardment. Controlled by Di, A0, B0, l0. Set to zero for evaporated films (P = 0).

For Modes 1 and 3 (stress-thickness), the model numerically integrates this rate equation from the starting thickness to the final thickness. For Mode 2 (steady-state stress), only Terms 1 and 3 are needed since grain size does not change.

## A.2  Key Parameters and Typical Ranges

| Parameter | Meaning | Typical range |
| --- | --- | --- |
| Sigma0 | Intrinsic tensile stress at coalescence (GPa) | 1–10 GPa |
| BetaD | Kinetic rate parameter controlling tension-compression transition (nm^2/s) | 0.01–5 |
| Ea | Activation energy for diffusion (eV) | 0–0.3 eV |
| Mfda | Biaxial modulus × lattice mismatch (GPa·nm) | 1–40 |
| L0 | Initial grain size at nucleation (nm) | 1–50 nm |
| alpha1 | Grain size growth rate with thickness (nm/nm) | 0.01–2 |
| GrainSize_200 | Grain size at 200 nm film thickness (nm) | 10–300 nm |
| SigmaC | Compressive stress offset (GPa) | −20 to 0 |
| K0 | Starting stress-thickness offset (N/m) | −5 to 5 |

## A.3  Early-State Model (Mode 4)

In the nucleation/coalescence regime, grains are modelled as ellipsoidal caps sitting on the substrate. The stress arises from the change in grain boundary area (dA) as adjacent grains impinge on each other. The key parameter is L_nul (grain nucleation size, nm). The model integrates the grain boundary area derivative (dA/dt) divided by the instantaneous grain size L(t) at each time step to accumulate kinetic stress.

Typical parameter ranges for supported materials:

| Material | Sigma0 | BetaD | Ea (eV) | Mfda | L_nul (nm) |
| --- | --- | --- | --- | --- | --- |
| Ag | ~5.0 | ~0.31 | ~0.06 | ~2.0 | 8–132 |
| Au | ~6.0 | ~0.65 | ~0.11 | ~16.0 | 32–78 |
| Ni | ~1.0 | ~2.27 | ~0.16 | ~8.6 | 0.7–1.5 |

Note: Ni has an unusually low Sigma0 (~1 GPa) and very small L_nul (~1 nm). Do not assume that parameter ranges from one material apply to another.

# References

[1]  T. Su, E. Chason, 'Analysis of early stage stress during Volmer-Weber growth of polycrystalline thin films,' Acta Materialia, vol. 301, 121534 (2025). DOI: 10.1016/j.actamat.2025.121534

[2]  E. Chason, T. Su, Z. Rao, 'Computational tool for analyzing stress in thin films,' Surface and Coatings Technology, vol. 474, 130099 (2023). DOI: 10.1016/j.surfcoat.2023.130099

[3]  T. Su, Z. Rao, S. Berman, D. Depla, E. Chason, 'Analysis of stress in sputter-deposited films using a kinetic model for Cu, Ni, Co, Cr, Mo, W,' Applied Surface Science, vol. 613, 156000 (2023). DOI: 10.1016/j.apsusc.2022.156000

[4]  E. Chason, A.M. Engwall, Z. Rao, T. Nishimura, 'Kinetic model for thin film stress including the effect of grain growth,' Journal of Applied Physics, 123, 185305 (2018).

[5]  E. Chason, J.W. Shin, S.J. Hearne, L.B. Freund, 'Kinetic model for dependence of thin film stress on growth rate, temperature, and microstructure,' Journal of Applied Physics, 111, 083520 (2012).

— End of User Manual —